Addendum to the Standalone API User's Guide

Verifying Correct Installation of the 

Noregon RP1210A-Compliant API

May 1, 2002

[image: image1.wmf]
Providing products and services to the transportation industry
Noregon Systems, Inc.

500 Shepherd Street, Suite 300

Winston-Salem, NC  27103

(336) 768-4337

FAX (336) 760-2540

E-MAIL: support@noregon.com

© Noregon Systems, Inc. 2002

Scope and Purpose

This document shows how to verify that the Noregon RP1210A-Compliant API is properly installed and is working in conjunction with one of the supported J1708 to RS232 conversion devices.  

Validation testing is performed using the Noregon “VB App”.  This application goes by the filename of “NSI32.EXE” and is located in the directory:  C:\Program Files\Noregon Standalone RP1210A API\Samples.  The example shows using the Noregon PLC/J1708 Adapter on COM1 (device identifier #15).

Selecting Interface Device and Connecting to the J1708 Data Bus (Figure 1, Steps 1-6)

1. Connect the interface device (adapter) to a known busy J1708 data bus and ensure that the polarity is correct (J1708+/J1708-) for the adapter.  If you are using a secured  (non-corporate) version of the API, you will need to place the hardware protect key (dongle) on the computers’ parallel port. 

2. Select the correct protocol, either J1708 or PLC (J1708 is used in the example).

3. Select the correct device identifier, the following is a list of device identifiers and their associated COM port:

	1
	Detroit Diesel "White Box" 
	COM1

	2
	Detroit Diesel "White Box" 
	COM2

	3
	Allison "Shaw Box" 
	COM1

	4
	Allison "Shaw Box" 
	COM2

	5
	Kent-Moore 
	COM1

	6
	Kent-Moore 
	COM2

	7
	B&B Electronics J1708P1 
	COM1

	8
	B&B Electronics J1708P1 
	COM2

	9
	B&B Electronics 232SAER 
	COM1


	10
	B&B Electronics 232SAER 
	COM2

	11
	Navistar 
	COM1

	12
	Navistar 
	COM2

	13
	Dana 
	COM1

	14
	Dana 
	COM2

	15
	Noregon PLC/J1708 Adapter 
	COM1

	16
	Noregon PLC/J1708 Adapter 
	COM2

	17
	B&B Electronics J1708D15 
	COM1

	18
	B&B Electronics J1708D15 
	COM2


4. Set the buffer size to 1000 to allow for large J1708 messages.

5. Connect to the data bus.

6. The Returned field should display 0.  If not, the API has not been installed properly.  Re-install and try again. 

Reading and Displaying of Messages and Bus Statistics (Figure 2, Steps 7-9)

7. You must issue the RP1210A command that sets filters to “Pass All” messages.  This allows messages to be read by the application. 

8. Set the application to read continuously.

9. Turn on monitoring of data bus statistics.

Sending and Verifying Sent Messages (Figure 3 and 4, Steps 10-13)

10. Build a J1708 message to send. 

11. Turn on RP1210A echoing so that the application will see the message as it is placed on the data bus.

12. Send the message repeatedly.

Pause reading continually to verify that the message is being transmitted.

Figure 1, Selecting Interface Device and Connecting to the J1708 Data Bus (Steps 1-6)


[image: image2.png](DLL-
Acton

Select
Protacel

- Connect\Disconnect

Device Type.

API-

[Eomes]| | cowa | FxS2o I Hwrd Diseld [o
{11708 =

B Twsz Retuned
5 CritPess E— fo

Send Message

@ sendlD:[5 [T2r282aates Send Cnd
rdCort | SendMsg | | Sendwiotiy [ IncDetaByte  J1708Fiaiy 7 =
e —I T~ Sendw/Block [~ Add.J1708 Checksum Remmedl— —
SendRae I 100 oncom 1~ fiop  _1708 | Clewser Discard
Set
fee Do
ResdCont| @ _Cear |
= Flaw ON
ReadMsg oot
I DiplyHes | Statistics
I~ BlockedFead | @ FXs Client Ut DA% MrtorStats Cooked
T~ Single Fead ¢ P Stats DTu% Cm—
Bulfer Size [1gog | TotalMsas. T Stats%. DAy [ _NewStats
Fetared [— | Bad Crkon) B M DPTx Resst || Retun
Calfsons TaMan Bus U Erm
I LogMessages | isg Rate T Full [~ iEics=tboln)
I LogStalisis | AvgLengh] Total Bytes R

Elapsed Time

~=lolx|

-Send Commands —
Command Number

Command Parameter

Reset Al

et Enor Description

EnCode





Figure 2, Reading and Displaying of Messages and Bus Statistics (Steps 7-9)

[image: image3.png](DLL-
Acton
- Connect\Disconnect

API-

~=lolx|

-Send Commands —

oot Comest || cata | F¥S2[5 I i Dise.1d [g | | CommendNumber
Fidoeo 11798 o] Tese [T [ Reared [5 —
Device Type  [75 _CritPass | Disconnect Command Parameler
~Send Message
@ sendlD:[5 [T2r282aates Send Cnd|
rdCort | SendMsg | | Sendwiotiy [ IncDetaByte  J1708Fiaiy 7 =
e —I T~ Sendw/Block [~ Add.J1708 Checksum Remmed’— —
SerdRtem) [100 oo 1 frog 1708 | _ClearSr Discard
Set
Reo 1D [122575253 142254 171100 006 000 051 127 043 000 080010 000 |
© [z o G K | e
22975241 142 040 132 085 001 184 000 000 Flaw ON
ReadMsg 422975225 122 032 000 130 000 000 —
L] 422975208 12 084 000 [ Echo OFF
I DisplpHen - Statistics
I~ BlockedFead | @ FXs Client Ut DA% MrtorStats Cooked
I” single Read Tis R Stals’ DTu% Cm—
Bulfer Sze [1og0 | TotalMsgs TuStats%. DPRy [ _NewStals
Bad Chism B M DPTx fesat || Petm [g
G ||| s TaMan Bus Ut I
I~ Logbossages | Mg R | ¥TeaFa [ s 8o |
I LogStalisis | AvgLengh] TotalBytes Rn | Resetl || Get Enor Desciption
Elapsed Time —
I EnCode





Figure 3 and 4, Sending and Verifying Sent Messages (Steps 10-13)

[image: image4.png]Construct A J1708 Message

Eter Number of Bytes InMessage [15
Message Byte to Use: [T

oK Cancel





[image: image5.png](DLL-

API-

=lalx]|
acton
- Connect\Disconnect ~Send Commands
ot Comeet | coval | FSz[E I A Disotd [5 | | Command Nurber
Potocal [11708 =] Tusz[; [ Retmed [g
DeviceType [75_CrttPass | Discornect Conmand Paramter
Sond essage
@ Sibi[; [zmzwERRRnE SendCnd
ot | Sendsp| T SendwMoiy [ incDalabye  J08Pioie [ || g |
e —I T~ Sendw/Block [~ AddJ1708 Checksum Remmed’n— —_—
gendfaem [100_ .o = g 2708 | _Clew Ser _Discad |
St
Rec. D] [s25740807 000 128 174 120 000 175 018 001 1
@ [T w0012 17 1A 008 7 oG oot E| ==
23140786 000 128 052 000130 000 000 FlowON
Read Mg §23140773 000 152084 000 —
L] 423140757 001 032 032 032 032 032 032 032 032 032 032 | EchoON
I DislayHex [ Statistios
I~ BlockedRead | @ FXs 007 CientUi [ 353 DRx% T MrtorStats Cooked
I Singe Fesd % oo FStt (170 DIk [ e —
BufeSie [iogn | TowlMsos | tosgs TuSta% | 0o DPRx [
Bad Chean) S PuMax [T70 DPTY [“05  Resm || Peum g
Rewred [T27 | Cofons | 19 Tublan o s [
I LogMessages | Msg Rate | g HTeRFul [ ==
I LogStatitis | Avalengh| 70 TowBytesR 40547 ResetAl || GetEnor Descrption
Elapsed Tine _
o034

EnCode [





2.  Select the J1708 Protocol.





4.  Set the buffer size to 1000.





3.  Select the correct device ID.





5.  Connect to the data bus.





8.  Turn on reading of messages.





9.  Monitor data bus statistics





13.  Pause the reading and verify that the message with all “32” bytes appears.





6.  Returned field should display 0.





7A.  Verify the Return field is zero.





7.  Set filtering to pass all messages.





12.  Send the message repeatedly.





12A.  Verify the return field is 0.





11.  Turn on RP1210A “Echoing”.  Press the button until it says “Echo On”.  The button changes its label between Off/On depending on the echoing state.





10.  Create a J1708 message.  This brings up the dialog box shown here on the left.  As the “Message Byte to Use”, input a value of 32 and then click “OK”.








2   
 


